ECE529 HW#3

Your Name:

Problem #1 (90 points)

Answer the review questions:

- How many independent Maxwell’s equations do we know?

- Why is Eq. (1a) called “Ampere’s law”?

- What is a simple solution to Maxwell’s equations with zero sources?

- What is the boundary condition on a surface of an ideal conductor (perfect metal)?

- What is the boundary condition on a surface of a real metal?

- Prove by direct substitution that Eqs. (4) give the solution to 1D Maxwell’s equations (3).

- List analogies between field and circuit parameters:

E-field –

H-field –

Dielectric permittivity –

Magnetic permeability –

- Describe in your own words the concept of “marching on in time” for the FDTD method.

- When you execute MATLAB script to example 1(fdtd1D_1.m), the field source is given by . On the other hand, you see on the screen a plane wave with the amplitude of 1V/m, but not of 2V/m as expected. Why is it so?

- Using MATLAB script to example 2 (fdtd1D_2.m) determine the amplitude of a standing voltage wave on the 600mm long transmission line when ,  (impedance mismatch). All other line parameters and frequency remain the same.

- Using MATLAB script to example 3 (fdtd1D_3.m) determine the power transmission coefficient at normal incidence when the target is a human body medium with the relative dielectric constant of 50 and the conductivity of 0.5S/m. All other script parameters including medium thickness and  frequency remain the same. Hint: Zoom in the MATLAB plot or change the scale in order to obtain sufficient resolution.

- Using MATLAB script to example 4 (fdtd3D_1.m) determine the power transfer function between two 15 cm long blade dipoles separated by 0.3m. All other script parameters and frequency remain the same.

Problem #2 (10 points)

Using MATLAB script fdtd1D_problem7.m (a slightly modified script to example 3)  determine the power transmission coefficient at normal incidence when the target is a glass wall in a conference room having thickness of 12.5mm (approximately ½”), relative dielectric constant of 4.0, and the conductivity of 0.01S/m. The signal frequency is 2.45GHz.

Hint: You have to carefully change all related script parameters and make sure that the script uses proper boundary conditions.